# 机组特点及概述

# Characteristics and Overview of Machine Set

# HMV150-800机组特点及概述：

# Characteristics and Overview of HMV150-800 Machine Set

1．概述Overview

150HMV150-800机组是我公司针对矿山排水而设计开发的机电一体化产品。该机组采用柴油机驱动，满足电力缺失工况。整体机组可以撬装，也可以放置浮体使用。性能优秀的泥泵设计，效率高，单泵扬程可达150米，排距远。采用电控控制的自动吸真空装置，启停方便。模块化设计，可根据用户要求，配置多品牌、多排放标准要求的柴油机。

150HMV150-800 machine set is a mechanical-electrical integration product, which is designed and developed by our company, especially used for mine drainage. This machine set is driven by diesel engine, guarantee the machine can still work under the lack of electricity circumstance. The whole machine set can be skid mounted, also can be placed on a floating body. This machine set is characterized by excellent mud pump design, high efficiency, 150 meters single stage head and long discharge distance. It is equipped with automatic vacuum suction device, which is controlled by electric, can make the start and stop much easier. The modular design, which makes the machine set could be configured with multiple brands, multiple emission standard diesel engines according to the requirements of users.

型号意义说明：Meaning of Model

HMV 150-800

150米扬程 150m Head，800m3/h流量800m3/h Flow

高压排水机组High Pressure drainage Unit

2．机组性能特点：Characteristics of Machine Set

150HX型排送泵：水力性能优秀，最大吸深8米，最大扬程150米，最高转速1400r/min。过流部件采用抗磨白口铸铁按照渣浆泵标准制造，可输送高浓度渣浆。

150HX Discharge Pump: It has excellent hydraulic performance, the maximum suction depth is 8 meters, the maximum head is 150 meters, the maximum speed is 1400r/min. The wet end parts are made of anti-wear white cast iron according to slurry pump standard, which can transport high concentration slurry.

- 过流能力强，可连续抽送砾石、高塑粘土块等；

- Strong flow capacity, can continuously pump gravel, high plastic clay blocks, etc.

过流能力主要取决于叶轮流道宽度。四枚叶片的叶轮，其每两叶片之间的过流宽度为55mm，可通过50mm左右的砾石和高塑粘土块。

The flow capacity mainly depends on the width of the impeller channel. The impeller with four vanes has a flow width of 55mm between each blade. The gravel and high plastic clay blocks with a diameter of 50mm can pass through.

- 配套性强，性能范围广，适用性强，可适用各种土质；

- Strong compatibility, wide range of performance, strong applicability and can be applied to all kinds of soil

高压输送泵常规配备四枚叶片叶轮，同时也可根据用户的具体要求，提供各种叶轮直径，各种叶片型式，适用于各种具体工况的叶轮。

High pressure discharge pump is usually equipped with four vanes impeller, but also can be equipped with impeller of various diameters and vane types according to the requirements of users, to suit different kinds of working duty and condition.

- 水力损失小，效率高，耗油量低；

- Low hydraulic loss, high efficiency, low fuel consumption

叶轮整体采用闭式叶轮，叶片型线采用多种匀变化曲线，叶轮前、后盖板和前、后护板表面均为机加工表面，泵体压水室采用螺旋线、等速度矩方法设计，可有效的减小水力损失，提高效率，从而降低耗油量，提高经济效益。

The impeller is closed type. The impeller vane profile adopts a variety of uniform change curves. The front and rear cover plate inserts of the impeller and the front and rear plates are with machined surfaces. The hydraulic pressure chamber of the pump body is designed by helix and equal velocity moment method, which can effectively reduce the hydraulic loss and improve efficiency, thus reducing fuel consumption and improving economic benefits.

3．船性特点：Characteristics of Structure

- 体积小、重量轻；Small Size and Light Weight

整机到每个零部件均采用无冗余设计，在保证性能及使用要求的前提下，尽量减少不必要的辅助结构与工艺结构，大大减小了整台泵的重量。

The pump and every part are designed without redundancy. The unnecessary auxiliary structure and process structure are minimized under the premise of ensuring performance and use requirements. Such design reduces the weight of whole pump greatly.

- 工作运行稳定性好，振动小，噪音低；

- Good working stability, low vibration, low noise

叶轮的静平衡要求按照G6.3级设计制作，同时降低泵运行的额定转速，在泵头前部设置支撑板结构，使得泵组能够平稳运转。The static balance of the impeller is designed in accordance with G6.3, while reducing the rated speed of pump operation, and keeping the pump operate steadily by setting a support plate structure in the front of the pump head.

- 结构简单可靠，易于拆装，维修方便；

- Simple and reliable structure, Easy to disassemble and maintain.

泵组的整体结构采用单级单吸悬臂式离心泵、前拆结构，拆装简单，维修方便，所有配合处均有起盖螺钉以利拆解；同时针对各个零部件的结构特点，配备专用拆装工具利于拆装；

The pump set adopts the single-stage single-suction cantilever centrifugal structure, and front disassembly structure. Simple to disassemble, easy to maintain. All the coordination points are equipped with lid screws to facilitate disassemble. At the same time, according to the structural characteristics of each component, special disassembly tools are designed to facilitate disassembly.

叶轮与轴之间采用螺纹进行联接，既能传递强大的扭矩，又便于拆下和装上；同时在轴套与迷宫套之间设有叶轮拆卸环，以备叶轮在难以拆卸时辅助拆卸。

The impeller and the shaft are connected with threads, which can not only transmit strong torque, but facilitate disassemble and installation; At the same time, the release collar is designed between the shaft sleeve and the labyrinth to prepare for the auxiliary disassemble of the impeller when it is difficult to disassemble.

4、经济效益特点Characteristics of Economic Benefit

- 零部件寿命长，相互之间匹配合宜；

- The parts have a long service life and are well matched with each other

泵的过流件、易磨损件采用我公司的耐磨系列材质，具有优良的抗磨损性能，在国内处于领先地位。叶轮、泵壳、前护板、后护板采用MS05材质，硬度可达57HRC以上,使用寿命长久。

Pump’s wet end parts and wear parts are made of our company's wear-resistant series of materials, with excellent anti-wear performance. This kind of material take the leading position in China. Impeller, casing, throat bush and frame plate liner insert are made of MS05 material, which hardness can reach to 57HRC or more, provide a long service life.

- 整机可靠性高，平均无故障 工作时间长；

- High reliability, No failures and long working hours

- 在整机设计过程中，充分考虑各个容易出现故障的部位，并对其进行改良设计，尽量提高整机的无故障工作时间。

In the whole design process of the machine, we fully considered the part which are prone to failure, then improve the design of those parts, try to improve the trouble-free working time of the whole machine as far as possible.

150HMV150-800柴油机组外形及结构由下图所示。

The outline and structure of the 150HMV150-800 diesel machine unit are shown in the following photo.